家用/商用服务机器人的安全

和EMC CR认证实施规则解读

1．适用范围

本认证规则适用于各类以帮助人类更简便、轻松生活为目的的服务机器人（参照GB/T12643-2013《机器人与机器人装备词汇》中的定义），主要包括个人/家庭类服务机器人（看家/清洁/做饭等）和商用/社会岗位类服务机器人（在非特殊领域帮助人类协同工作的服务机器人，如主持/接待/收银/导购/监考/送餐等）两种类型，为确保人类与这类机器人能够实现近距离人机互动甚至人机接触等进行的基本安全设计、防护措施、使用信息要求、产品责任、伦理与保险及可靠性等方面的安全和电磁兼容（EMC）认证。
    本认证规则暂不适用于以下几类特殊领域的服务机器人：

医疗服务机器人

个人护理服务机器人

民用无人机

国家规定的其它专业领域类服务机器人。

其中个人护理服务机器人和民用无人机可分别依据《个人护理机器人的安全和EMC 认证实施规则》和《民用无人机的安全和EMC 认证实施规则》进行安全和电磁兼容（EMC）认证工作。

- 认证模式

- 生产企业可以按照表1 选择适宜的认证模式：

表 1-认证模式分类

| 认证模式 | 认证模式说明 |
| --- | --- |
| 模式 A：产品型式试验 | 证书仅对单个产品有效（在证书上增加产品的序列 号），不规定证书有效期限。 |
| 模式 B：产品型式试验+初次工厂检查+ 获证后监督 | 产品证书为 5 年，有效期内的证书有效性依据获证后 的监督维持。 |

认证的基本环节包括：

- 认证的申请

- b) 型式试验

- c) 初始工厂检查（模式B 适用）

- d) 认证结果评价与批准

- e) 获证后的监督（模式B 适用）

- f) 复审

- 3．认证申请

- 申请人申请认证时可以选择安全认证、电磁兼容认证或安全+电磁兼容认证。

- 3.1 认证单元划分

- 原则上以同一生产厂的同一基本型号为一个认证申请单元。不同的生产场地的产品为不同的申请单元，型式试验仅在一个生产企业的样品上进行，必要时，其他生产企业应提供样品和相关资料供认证机构进行一致性核查。

- b) 产品内部或系统电气结构和对产品安全和EMC 性能有重要影响的元件相同的整机，可作为一个单元申请认证。
3.2 申请认证提交资料

- 3.2.1 申请资料

- 正式申请书(网络填写申请书后打印或下载空白申请书填写)

- b) 工厂检查调查表（模式B 首次申请时）

c) 产品描述

3.2.2 证明资料

- 委托人、生产者、生产企业的注册证明如营业执照、组织机构代码（首次申请时）

- b) 委托人为销售者、进口商时，还须提交销售者和生产者、进口商和生产者订立的相关合同副本

- c) 代理人的授权委托书（如有）

- d) 有效的监督检查报告或工厂检查报告（如有）

e) 其他需要的文件

3.2.3 提供与产品有关的资料

- 产品总装图、电器原理图、产品说明书等

- b) 电参数表

- c) 关键零部件/元器件清单

- d) 同一申请单元内各个型号产品之间的差异说明

e) 无线电型号核准证书（如产品带有无线电发射功能）

- 型式试验

- 4.1 样品

- 4.1.1 申请单元中只有一个型号的，选该型号为样品

以系列产品为同一申请单元申请认证时，样品应从系列产品中选取具有代表性的型号，并且选取的样品应尽可能覆盖系列产品的安全和EMC 要求，不能覆盖时，还应选取申请单元内的其他型号样品做补充差异试验。

4.1.2 样品数量

申请单元代表性型号样品数量为1 套，补充试验样品数量视代表性型号样品覆盖申请单元内产品的安全和EMC 要求的实际情况而定，代表性型号样品与补充试验样品在能覆盖申请单元内系列产品安全和EMC 要求的前提下，应尽可能减少补充试验样品数量和补充试验项目。

4.1.3 样品及资料处置

试验结束并出具试验报告后，有关试验记录和相关资料由检测机构保存，样品由认证机构和企业协商处置。

4.2 型式试验

4.2.1 依据标准

申请方应按照下表列出的标准申请产品认证：

- 对应国内销售的机器人认证：

| 序号 | 类别 | 标准编号 | 标准名称 | 备注 |
| --- | --- | --- | --- | --- |
| 1 | 安全 | GB 5226.1-2008 | 机械电气安全 机械电气设备 第 1 部 分：通用技术条件 | 均适用 |
| 2 |  | GB/T 15706-2012 | 机械安全 设计通则 风险评估与风险 减小 |  |
| 3 |  | GB 28526-2012 | 机械电气安全 安全相关电气、电子和 可编程电子控制系统的功能安全 | 可任选一 份标准进 行功能安 全评估 |
|  |  | GB/T 16855.1-2008 | 机械安全 控制系统有关安全部件 第 1 部分：设计通则 |  |
| 4 |  | GB 4943.1-2011 | 信息技术设备 安全 第 1 部分：通用 要求 | 均适用 |
| 5 | EMC | GB/T 17799.1-1999 | 电磁兼容 通用标准 居住、商业和轻 工业环境中的抗扰度试验 | 均适用 |
| 6 |  | GB 17799.3-2012 | 电磁兼容 通用标准 居住、商业和轻 工业环境中的发射 |  |

（2）对应出口销售的机器人认证（IEC 标准）：

| 序号 | 类别 | 标准编号 | 标准名称 | 备注 |
| --- | --- | --- | --- | --- |
| 1 | 安全 | IEC 60204-1:2005 +A1:2008 | Safety of machinery - Electrical equipment of machines - Part 1: General requirements | 均适用 |
| 2 |  | ISO 12100:2010 | Safety of machinery -- General principles for design -- Risk assessment and risk reduction |  |
| 3 |  | IEC 62061:2005+A1:2012+ A2:2015 | Safety of machinery - Functional safety of safety-related electrical, electronic and programmable electronic control systems | 可任选一 |
|  |  |  |  | 份标准进 行功能安 |
|  |  | ISO 13849-1:2015 | Safety of machinery -- Safety-related parts of control systems -- Part 1: General principles for design |  |
|  |  |  |  | 全评估 |
| 4 |  | IEC 60950-1:2005+A1： 2009+A2:2013 | Information technology equipment - Safety - Part 1: General requirements | 均适用 |
| 5 | EMC | IEC 61000-6-1:2005 | Electromagnetic compatibility (EMC) - Part 6-1: Generic standards - Immunity for residential, commercial and light-industrial environments | 均适用 |
| 6 |  | IEC 61000-6-3:2006 +A1:2010 | Electromagnetic compatibility (EMC) - Part 6-3: Generic standards - Emission standard for residential, commercial and light-industrial environments |  |

（3）对应出口销售的机器人认证（EN 标准）：

| 序号 | 类别 | 标准编号 | 标准名称 | 备注 |
| --- | --- | --- | --- | --- |
| 1 | 安全 | EN 60204-1:2006 +A1:2009 | Safety of machinery - Electrical equipment of machines - Part 1: General requirements | 均适用 |
| 2 |  | EN ISO 12100:2010 | Safety of machinery - General principles for design - Risk assessment and risk reduction |  |
| 3 |  | EN 62061:2005+A1:2013+ A2:2015 | Safety of machinery - Functional safety of safety-related electrical, electronic and programmable electronic control systems | 可任选一 份标准进 行功能安 全评估 |
|  |  | EN ISO 13849-1:2015 | Safety of machinery - Safety-related parts of control systems - Part 1: General principles for design |  |
| 4 |  | EN 60950-1:2006+A1： 2010+A2:2013 | Information technology equipment - Safety - Part 1: General requirements | 均适用 |
| 5 | EMC | EN 61000-6-1:2007 | Electromagnetic compatibility (EMC) - Part 6-1: Generic standards - Immunity for residential, commercial and light-industrial environments | 均适用 |
| 6 |  | EN 61000-6-3:2007 +A1:2011 | Electromagnetic compatibility (EMC) - Part 6-3: Generic standards - Emission standard for residential, commercial and light-industrial environments |  |